ECE 233 Project Report

Hybrid Digital and Analog Beamforming Design

for Large-Scale Antenna Arrays

Ruoye Wang | 605625594

Yida Chen | 005852117

Abstract

Abstract

TODO change figure number

System Model

This project implements a model proposed by the reference paper. This narrowband downlink single-cell multi-user MIMO system model has a two-stage hybrid digital and analog beamforming architecture at the base station (BS) and the user terminals.

Figure 1: Block diagram of the model proposed by the paper

As the figure above shows, the BS has N antennas and NtRF RF chains and serves K users. Each user is equipped with M antennas and NrRF RF chains and requires d data streams. The number of data streams required by each user d≤NrRF≤M; the total number of data streams NS=Kd≤NtRF≤N.

This project assumes single-user scenario, i.e., K=1. To simplify the notation while preserving the generality, it is assumed that NtRF=NrRF=NRF. The project first implements the hybrid beamformer design for the case where NRF=NS to show that, according to Proposition 1, a fully digital beamformer architecture can be realized by a hybrid structure with at least NS RF chains using a proposed heuristic algorithm. Then the same algorithm is implemented for the case where NS<NRF<2NS.

The symbols VD,  VRF,  WD,  WRF  represent the digital precoder at the BS (size NtRF×NS), the RF precoder at the BS (size N×NtRF), the digital combiner at the user end (size NrRF×NS), and the RF combiner at the user end (size M×NrRF). H (size M×N) is the matrix of the complex channel gains from the transmit antennas of the BS to the user (note that since K=1, all Hk can be represented by a single H; the same can be applied to other user-specific quantities in the paper).

Then, the transmitted signal can be represented by:

Figure 2: Transmitted signal

Where s is the vector of data symbols. And the final processed signals are obtained as:

Figure 3: Final processed signal

Where zk represents the additive white Gaussian noise.

Main Part

This paper mainly focuses on maximizing the overall spectral efficiency with total transmit power constrained and H fully known. This requires us to find the optimal solution for precoders at the transmitter end and the combiners at the receiver end, which can be represented by this formula:

Figure 4: the formula representing the main problem (K=1, disregarding k)

This formula is calculated under the aforementioned cases NRF=NS and NS<NRF<2NS. It can be simplified for precoder design:

Figure 5: Precoder design formula

NRF=NS scenario

When NRF=NS, VRF can be calculated by:

Figure 6: RF precoder design formula

Figure 7: Algorithm 1 to calculate RF precoder, with infinite resolution phase shifter

Which is a simplification of Figure. 5 assuming VDVDH≈γ2I. It is summarized in Algorithm 1:

The formula to calculate VRFi,j is under the assumption of infinite phase shifters. Since accurate phase shifters can be expensive to implement in real world, they are commonly replaced by cost effective low resolution phase shifters. To simulate such phase shifters, the design of VRF is quantized:

Figure 8: RF precoder design, with 1-bit resolution phase shifter

Then VD can be calculated by solving:

Figure 9: Digital precoder design formula

Where Q=VRFHVRF and Heff=HVRF.

We use a water-filling solution (Reference: Waterfilling) to allocate power Pi to each channel so that the overall channel capacity is maximized. The sum of power Pi satisfies:

Figure 10: Constraint on power allocation

Where the power of the first m most efficient channels sum up to use P to the largest extend. Since power cannot be negative:

Figure 11: Formula to calculate allocated power

Figure 12: Formula to calculate water level

Combining the above two formulas, μ the water level chosen to satisfy the power sum constraints with equality can be represented by:

m is obtained by iterating through the number of total channels NS. In each iteration, μ  is calculated using the above formula, and compared with σ2hi, where σ2=PSNRlinear  represents noise and hi is the square of right singular value of the SVD result of HeffQ-1/2, since in Figure 9, the matrices are applied twice on both left and right. Similarly, the diagonal elements of Γe are all square roots of Pi. (Reference: Ruifu Donar Li)

The final value of m is chosen so that μ satisfies the power constraint, as the following figure shows:

Figure 13: Water-filling solution

Then the matrix of digital precoder can be calculated using the formula proposed by the paper:

VD=Q-1/2UeΓe

Where Ue is the set of right singular vectors corresponding to the NS largest singular values of HeffQ-1/2 and Γe is the diagonal matrix of allocated powers to each stream.

With VD,  VRF designed, the formula in Figure 4 can be simplified to:

Figure 14: RF combiner design formula

Due to its similarity to Figure 6, it can also be solved with Algorithm 1, with F1 replaced by F2=HVtVtHHH, Vt=VRFVD and (WRFHWRF)-1=1M since M is large. The quantized WRF is also calculated with the same method as VRF.

Finally, the optimal digital combiner can be obtained from its MMSE solution:

Figure 15: Digital combiner design formula

After VD,  VRF,  WD,  WRF have been obtained, the performance of the model can be evaluated by its spectral efficiency:

Figure 16: Formula of spectral efficiency

The above process is repeated for a range of SNR, and for each SNR value, spectral efficiency is averaged over 100 Monte Carlo trials.

NS<NRF<2NS scenario

As stated in the paper, this scenario can still be implemented using the aforementioned process. It is implemented for NRF={NS,  NS+1, NS+3}.

Results and Discussion

Figure 17: Spectral efficiency vs. SNR for result 1

Plot the spectral efficiency vs. SNR in the range −10 dB to 6 dB, assuming a 64 × 16 MIMO system and NRF=NS=6.

The resulting figure is similar to Fig. 2 in the paper, with ~1.5dB difference on SNR values.

Figure 18: Spectral efficiency (infinite resolution and 1-bit phase shifters) vs. SNR for result 2

Plot the spectral efficiency vs. SNR in the range 0 dB to 30 dB, assuming a 10 × 10 MIMO system, NRF=NS=2, and phase shifters with 1-bit and infinite resolutions.

In paper Fig. 3, the spectral efficiency of 1-bit phase shifter is higher than that of infinite resolution phase shifter. However, this resulting figure shows the opposite. One possible reason is that, the paper’s beamformer is designed under the assumption of infinite resolution phase shifters and each entry of the RF beamformer is quantized to the nearest point of the set of possible phases. However, in this project, the beamformers for infinite resolution phase shifter is not quantized.

- Plot the spectral efficiency vs. SNR in the range −10 dB to 6 dB, assuming a 64 × 16 MIMO system, NS=4, NRF={NS,  NS+1, NS+3}, and phase shifters with 1-bit and infinite resolutions.

Figure 20: Spectral efficiency (1-bit phase shifter) vs. SNR, 100 Monte Carlo trials

Figure 19: Spectral efficiency (Infinite resolution phase shifter) vs. SNR, 100 Monte Carlo trials

The curves for 1-bit phase shifter are ~1.5dB lower than paper Fig. 4 which is considered acceptable error. Same as Fig. 4, the curves for infinite phase shifters (plotted separately for clarity) are significantly higher than that of 1-bit phase shifters, and the spectral efficiency increases as the number of RF chains increase, so that 1-bit phase shifter with more RF chains has a performance close to infinite phase shifters. This is in accordance with the paper’s conclusion that the number of RF chains can be used to trade off the accuracy of phase shifters in hybrid beamforming design.

Figure 21: Spectral efficiency (infinite resolution phase shifter) vs. SNR, 1000 Monte Carlo trials

The above curves are rather unsmooth because the number of Monte Carlo trials are small. After the number of trials is increased to 1000, the figures become:

Figure 22: Spectral efficiency (1-bit phase shifter) vs. SNR, 1000 Monte Carlo trials

Which more clearly shows the paper’s discoveries.

Conclusion

This project implements the hybrid beamformer architecture proposed by the reference paper for wireless systems with large-scale antenna arrays. It focuses on single-user scenario and explores the performance of hybrid beamformers. It first implements a hybrid beamformer design in a 64*16 MIMO system assuming the number of RF chains is equal to the number of data streams, then modifies the algorithm to compare the difference between infinite resolution and 1-bit phase shifters. The process is repeated for the scenario that the number of RF chains is less than twice but larger than once of the number of data streams. The simulation results are in accordance with the paper’s simulation figures with only minor difference on the range of spectral efficiency. Result 2 shows the performance of infinite resolution phase shifter is better than that of 1-bit phase shifter, which is opposite from paper Fig. 3, because the project does not quantize the infinite resolution phase shifter as in the paper. Result 3 shows that increasing the number of RF chains in hybrid beamformer architectures trades off its insufficiency in accuracy compared to fully digital ones and thus allows it to serve as a sub-optimal solution for massive MIMO systems.